XIAO ESP32S3 Camera — Test Results

Updated 20 May 2026  ·  Firmware flash 2026-05-19

──────────────────────────────────────────────────────────────────────────

| Camera IP | Board | Test Date | Firmware Flash Date |
| --- | --- | --- | --- |
| 192.168.0.39 (home) / 10.10.10.155 (work) | Seeed XIAO ESP32S3 Sense (OV2640) | 2026-05-19 | 2026-05-19 |

Firmware updated 2026-05-19: multi-network WiFi (SenSen2 + Home2.4g), 5 s capture cooldown added, baseline captured at boot, FPS now reported as lifetime average (totalFramesSent / uptime_ms). SXGA and UXGA resolution switch causes board crash in new firmware — under investigation; old data used for those rows.

| Test | Method | Result |
| --- | --- | --- |
| T1 — Motion detection | Semi-automated (--motion flag) | Pending — PIR attached, cooldown now fixed |
| T2 — WiFi range | Manual | Not yet run |
| T3 — Face recognition dist. | Manual | PASS (2026-05-14, unchanged) |
| T3b — Resolution vs FPS | Automated | PASS — VGA optimal (25 fps) |
| T4 — Power / temps | Partially automated | PASS (RTSP & PIR rows blank) |
| T5 — Multi-stream | Automated | PASS — all 5 clients alive |
| T6 — RTSP | Partially automated | PASS (VLC steps manual) |
| T6b — 5-min stability | Automated | PASS |
| TEP — Endpoint check | Automated | PASS |
| T7 — Multiple cameras | Manual | Not yet run — needs 2nd unit |

# T1 — Motion Detection & Capture Latency  [Pending]

Method: python run_tests.py 192.168.0.39 --motion — polls /stats for 30 s, logs latency and in-frame status per capture. Wave hand at PIR sensor on D0. PIR wiring: red→3V3, black→GND, yellow→D0. Run with no other browser tabs open.

| Trial | Latency (ms) | JPEG size (bytes) | In-frame? | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Implications

5-second cooldown now prevents hold-time re-triggering — one walk-through should give exactly 1 capture, not 2–3.

If consistently off-camera, PIR detects movement outside camera FoV — aim both sensors in same direction.

Latency >500 ms means camera buffer was held by a concurrent stream; close all streams before this test.

Notes:

# T2 — WiFi Range  [Not yet run]

Method: Manual — move camera to each location on battery bank, wait 10 s, run: python wifi_test.py http://<ip> "location". Note: IP arg now requires http:// prefix.

| Location | Dist. (m) | Walls | RSSI (dBm) | FPS — 1 viewer | FPS — 2 viewers |
| --- | --- | --- | --- | --- | --- |
| Next to router |  | None |  |  |  |
| Same room, far end |  | — |  |  |  |
| Adjacent room |  | 1 |  |  |  |
| Two rooms away |  | 2 |  |  |  |

RSSI guide: >−60 good | −70 acceptable | <−75 expect drops

## Implications

Below −70 dBm FPS drops significantly — this is the practical deployment range limit.

Each wall costs roughly 10–15 dBm; reposition router before moving camera if signal is marginal.

FPS from /stats is a lifetime average — comparison between 1-viewer and 2-viewer rows is still meaningful (both equally averaged), but absolute values are not comparable to old tests.

Notes:

# T3 — Face Recognition Distance  [PASS — 2026-05-14, unchanged]

Method: Manual — subject walked away in a straight corridor under fair indoor lighting. Observer called clear / barely / gone; distances measured with tape measure. Repeated per resolution. Fair lighting, 30 fps.

| Resolution | Clearly recognisable (cm) | Barely recognisable (cm) | Not recognisable (cm) |
| --- | --- | --- | --- |
| 240p (320×240) | 100 | 150 | 200 |
| 480p (640×480) | 150 | 200 | 250 |
| 720p (1280×720) | 300 | 350 | 400 |
| SXGA (1280×1024) | 320 | 360 | 400 |
| UXGA (1600×1200) | 380 | 450 | 500 |

## Implications

720p is the recommended resolution for a 3 m corridor — faces clearly recognisable at 3 m, still visible to 4 m.

UXGA adds only ~80 cm over 720p at cost of lower FPS; only use it for corridors longer than ~3.5 m.

All thresholds drop 30–50% in dim lighting — add supplementary lighting or switch to IR camera for dark environments.

Notes:

# T3b — Resolution vs FPS Trade-off  [PASS]

Method: Automated — res_fps_test.py. For each resolution: sends GET /res?id=<id>, waits 5 s (SETTLE_SECS), then opens one HTTP MJPEG stream. Raw bytes collected for 12 s; counts --frame boundary markers. No FPS cap. One sample JPEG per resolution saved to ./frames/. 2026-05-19. Board cold (~47–52°C).

| Resolution | Dimensions | FPS (2026-05-19) | FPS (2026-05-14) | Temp (°C) | Face range (T3) | >=20 fps? |
| --- | --- | --- | --- | --- | --- | --- |
| qvga | 320×240 | 26.5 | 25.4 | 47.3 | 100 cm | YES |
| vga | 640×480 | 25.0 | 24.8 | 50.3 | 150 cm | YES — recommended |
| svga | 800×600 | 13.3 | 13.1 | 50.3 | — | NO |
| hd | 1280×720 | 13.5 | 13.9 | 52.3 | 300 cm | NO |
| sxga | 1280×1024 | — | 10.6 | — | 320 cm | NO |
| uxga | 1600×1200 | — | 7.8 | — | 380 cm | NO |

SXGA and UXGA: board crashes (resets) when switching to these resolutions in new firmware. Temp drops from 52°C to 43°C at SXGA switch = camera stopped. Old data (2026-05-14) used. Needs investigation before deploying at high resolution.

## Implications

Hard cliff at VGA confirmed: qvga and vga deliver ~25 fps; everything above VGA drops to 13 fps. OV2640 hardware limit.

Recommended for motion capture: VGA (640x480) — highest resolution sustaining >=20 fps.

SVGA result (13.3 fps) confirmed consistent across both firmware versions — cliff is reproducible.

SXGA+ crash is a regression in new firmware. Do not deploy at SXGA/UXGA until fixed.

Notes:

# T4 — Power & Temperature  [PASS — partial]

Method: Idle row automated. Stream rows captured during T5. RTSP and PIR rows require manual INA219 measurement. FPS column is lifetime average (totalFramesSent / uptime_ms) — not per-state current FPS. 2026-05-19.

| State | CPU | Est. mA | Actual mA (INA219) | Temp (°C) | FPS (lifetime avg) |
| --- | --- | --- | --- | --- | --- |
| Idle | 240 MHz | ~310 |  | 39.3 | 0.3 |
| 1x MJPEG stream | 240 MHz | ~310 |  | 42.3 | 0.4 |
| 2x MJPEG streams | 240 MHz | ~310 |  | 43.3 | 0.7 |
| 3x MJPEG streams | 240 MHz | ~310 |  | 44.3 | 0.9 |
| 4x MJPEG streams | 240 MHz | ~310 |  | 45.3 | 1.1 |
| 5x MJPEG streams | 240 MHz | ~310 |  | 46.3 | 1.2 |
| 1x RTSP (VLC) |  |  |  |  |  |
| PIR during stream |  |  |  |  |  |

FPS column shows lifetime average since boot — dragged down by idle time before T5. Not comparable to old T4 FPS numbers. Use T3b (res_fps_test.py) for accurate per-resolution FPS.

## Implications

Temperature rises ~7°C from idle to 5-stream load — passive cooling sufficient in normal indoor environments.

Board at home runs 7–10°C cooler than at work (stronger signal = less radio power = less heat).

For accurate battery life estimate, clip INA219 on 3.3 V rail — firmware estimate is not load-sensitive.

Notes:

# T5 — Multiple Simultaneous Streams  [PASS]

Method: Automated — opens 1–5 HTTP MJPEG streams in threads, waits 5 s per step, reads /stats. Frames rx column is cumulative total across all clients since test started — NOT per-step FPS. Only the 1-client row gives a clean per-client rate. 2026-05-19.

| Clients | All alive? | Frames rx (cumulative) | Temp (°C) | Crashes |
| --- | --- | --- | --- | --- |
| 1 | Yes | 102 | 42.3 | None |
| 2 | Yes | 238 | 43.3 | None |
| 3 | Yes | 409 | 44.3 | None |
| 4 | Yes | 541 | 45.3 | None |
| 5 | Yes | 657 | 46.3 | None |

1-client clean rate: 102 frames / 5 s = ~20 fps. Beyond row 1, Frames rx includes all prior clients still running — rows cannot be compared directly. All-alive column is the meaningful pass/fail metric.

## Old T5 data (2026-05-14) for reference — FPS measured per-step (old firmware)

| Clients | FPS (per-step, old method) | Temp (°C) | Crashes |
| --- | --- | --- | --- |
| 1 | 27.8 | 51.3 | None |
| 2 | 12.4 | 53.3 | None |
| 3 | 7.3 | 53.3 | None |
| 4 | 6.3 | 53.3 | None |
| 5 | 5.6 | 54.3 | None |

Old FPS numbers were from a per-handler racy global (streamFps) — values with 2+ clients are unreliable. New firmware removes the race but the test script needs updating to compute per-step delta for clean multi-client FPS.

## Implications

For recognition pipeline use 1 client only — gives ~20–28 fps depending on firmware and network.

Use RTSP for recognition software and HTTP /stream for human preview only.

Temperature at 5 clients (46°C) well below 85°C throttle threshold.

Notes:

# T6 — RTSP Stream  [PASS — VLC steps manual]

Method: Automated OPTIONS probe (raw TCP socket to :8554). VLC visual checks are manual. 2026-05-19.

RTSP URL: rtsp://192.168.0.39:8554/mjpeg/1  (home) / rtsp://10.10.10.155:8554/mjpeg/1  (work)

| Step | Result |
| --- | --- |
| RTSP OPTIONS probe | PASS — RTSP/1.0 200 OK (2026-05-19) |
| Methods advertised | DESCRIBE, SETUP, TEARDOWN, PLAY, PAUSE |
| VLC connects and plays |  |
| No green bands / duplication |  |
| Resolution change visible in VLC |  |
| Quality change visible in VLC |  |

## Implications

Only 1 simultaneous RTSP client supported — recognition software should be sole RTSP consumer.

Resolution changes apply immediately via /res; RTSP reflects new resolution on next VLC reconnect.

No authentication — any device on the same network can change settings.

Notes:

# T6b — 5-Minute Stability  [PASS]

Method: Automated — 3 simultaneous HTTP streams held for 300 s, polled every 30 s. 2026-05-19. FPS is lifetime average — jitter reflects dilution by earlier idle time, not real FPS instability.

| Elapsed (s) | Streams alive | FPS (lifetime avg) | Temp (°C) | Free RAM | RSSI (dBm) |
| --- | --- | --- | --- | --- | --- |
| 30 | 3/3 | 2.3 | 46.3 | 189 / 323 KB | -51 |
| 60 | 3/3 | 3.3 | 47.3 | 188 / 323 KB | -51 |
| 90 | 3/3 | 3.8 | 46.3 | 181 / 323 KB | -54 |
| 120 | 3/3 | 4.5 | 48.3 | 188 / 323 KB | -54 |
| 150 | 3/3 | 5.1 | 49.3 | 188 / 323 KB | -59 |
| 180 | 3/3 | 5.6 | 48.3 | 188 / 323 KB | -51 |
| 210 | 3/3 | 6.0 | 51.3 | 181 / 323 KB | -54 |
| 241 | 3/3 | 6.4 | 49.3 | 190 / 323 KB | -53 |
| 271 | 3/3 | 6.8 | 50.3 | 188 / 323 KB | -57 |
| 301 | 3/3 | 7.4 | 53.3 | 188 / 323 KB | -52 |

PASS — all 3 streams alive throughout. Temp plateaued at 53°C (safe limit 85°C). RAM stable, no leak. RSSI -51 to -59 dBm (good). Board runs cooler at home vs work (-38 dBm work vs -51 to -59 dBm home).

Rising FPS from 2.3 to 7.4 over 5 min is expected with the lifetime average formula — it's approaching the true streaming rate as idle time becomes a smaller fraction of total uptime. Not a sign of instability.

## Old T6b data (2026-05-14) for reference

| Elapsed (s) | Streams alive | FPS (/stats, old) | Temp (°C) | Free RAM | RSSI (dBm) |
| --- | --- | --- | --- | --- | --- |
| 30 | 3/3 | 9.2 | 56.3 | 190 / 323 KB | -38 |
| 60 | 3/3 | 6.5 | 57.3 | 183 / 323 KB | -38 |
| 120 | 3/3 | 6.5 | 59.3 | 191 / 323 KB | -39 |
| 180 | 3/3 | 6.5 | 60.3 | 182 / 323 KB | -38 |
| 300 | 3/3 | 7.0 | 62.3 | 191 / 323 KB | -38 |

Old FPS from /stats used the racy per-handler global — values were unreliable under 3-client load. Old temps higher (56–62°C) because board ran at work (weaker signal = more radio power).

## Implications

System can run indefinitely — no memory leak detected.

Chip reaches thermal equilibrium through passive cooling alone — no heatsink needed indoors.

Notes:

# TEP — Endpoint Verification  [PASS]

Method: Automated — sends GET to each control endpoint, re-reads /stats to confirm change, then restores defaults. 2026-05-19.

| Endpoint | Value set | Confirmed in /stats | Result |
| --- | --- | --- | --- |
| /res?id=qvga | qvga (320x240) | resolution = qvga | PASS |
| /quality?q=10 | q=10 (Best) | quality = 10 | PASS |
| /fps?delay=33 | 33 ms delay | frame_delay = 33 | PASS |

# T7 — Multiple Cameras  [Not yet run — requires second unit]

Method: Manual — flash second board with same firmware, connect both to WiFi, verify independent stream and control access. Note: test scripts must have IP passed as argument (no hardcoded IP) before running T7.

| Test | Camera 1 | Camera 2 |
| --- | --- | --- |
| Both streaming simultaneously |  |  |
| Both RTSP in VLC simultaneously |  |  |
| Res change on one does not affect other |  |  |
| Motion detection independent |  |  |

──────────────────────────────────────────────────────────────────────────

Board: Seeed XIAO ESP32S3 Sense  |  Home IP: 192.168.0.39  |  Work IP: 10.10.10.155  |  Updated: 2026-05-20