XIAO ESP32S3 Camera Test Results

Updated 21 May 2026

──────────────────────────────────────────────────────────────────────────

| Camera IP | Board | Last Test Date | Firmware Flash Date |
| --- | --- | --- | --- |
| 10.10.10.155 (work) / esp32cam.local (hybrid/deepsleep) | Seeed XIAO ESP32S3 Sense (OV2640) | 2026-05-21 | 2026-05-21 |

Three firmware environments: 'streaming' (always-on MJPEG+RTSP), 'deepsleep' (PIR wake → 1 photo → sleep), 'hybrid' (PIR wake → 30s MJPEG stream → sleep). Upload with: pio run -e <streaming|deepsleep|hybrid> --target upload. SXGA and UXGA resolution switch crashes streaming firmware — under investigation.

| Test | Method | Result |
| --- | --- | --- |
| T1 — Motion detection | Semi-automated (--motion flag) | Pending — PIR attached, cooldown not yet applied |
| T2 — WiFi range | wifi_test.py | PASS (2026-05-21, SenSen2) |
| T3 — Face recognition dist. | Manual | PASS (2026-05-14, unchanged) |
| T3b — Resolution vs FPS | res_fps_test.py | PASS VGA optimal (25 fps) |
| T4 — Power / temps | Partially automated | PASS (RTSP & PIR rows blank) |
| T5 — Multi-stream | Automated | PASS all 5 clients alive |
| T6 — RTSP | Partially automated | PASS (VLC steps manual) |
| T6b — 5-min stability | Automated | PASS |
| TEP — Endpoint check | Automated | PASS |
| TM — Motion capture (streaming) | motion_test.py | PASS all 4 subjects, face visible |
| TH — Motion capture (hybrid) | motion_test.py + hybrid firmware | PASS all 4 subjects on battery |
| TD — Deep sleep single photo | motion_test_deepsleep.py | Partial 1 photo/wake insufficient |
| T7 — Multiple cameras | Manual | Not yet run — needs 2nd unit |

# T1 Motion Detection & Capture Latency  [Pending]

Method: python run_tests.py 192.168.0.39 --motion — polls /stats for 30 s, logs latency and in-frame status per capture. Wave hand at PIR sensor on D0. PIR wiring: red→3V3, black→GND, yellow→D0. Run with no other browser tabs open.

| Trial | Latency (ms) | JPEG size (bytes) | In-frame? | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Implications

- 5-second cooldown now prevents hold-time re-triggering one walk-through should give exactly 1 capture, not 2–3.

- If consistently off-camera, PIR detects movement outside camera FoV aim both sensors in same direction.

- Latency >500 ms means camera buffer was held by a concurrent stream; close all streams before this test.

# T2 WiFi Range  [PASS]

Method: wifi_test.py 10 s frame count per viewer at each location. Network: SenSen2.

| Location | RSSI (dBm) | FPS 1 viewer | FPS each 2 viewers |
| --- | --- | --- | --- |
| Next to router | −39 | 22.5 | 12.5 |
| Other side of same room | −51 | 24.2 | 12.2 |
| Next room (1 wall) | −58 | 23.2 | 10.2 |
| Outside building (same floor) | −76 | 8.1 | 3.9 |
| Outside building | −86 | 3.5 | 1.8 |
| Across the street | — | Unreachable | — |

RSSI guide: above −60 dBm = strong | −70 dBm = acceptable | below −75 dBm = degraded

## Implications

- Building wall is a hard boundary inside signal stays above −60 dBm and FPS above 20. Outside, both collapse.

- Usable streaming (>10 fps/viewer) cuts off at the building wall.

- Each wall costs roughly 10–15 dBm; reposition router before moving camera if signal is marginal

# T3 Face Recognition Distance  [PASS]

Method: Manual subject walked away in a straight corridor under fair indoor lighting. Observer called clear / barely / gone; distances measured with tape measure. Repeated per resolution. Fair lighting.

| Resolution | Clearly recognisable (cm) | Barely recognisable (cm) | Not recognisable (cm) |
| --- | --- | --- | --- |
| 240p (320×240) | 100 | 150 | 200 |
| 480p (640×480) | 150 | 200 | 250 |
| 720p (1280×720) | 300 | 350 | 400 |
| SXGA (1280×1024) | 320 | 360 | 400 |
| UXGA (1600×1200) | 380 | 450 | 500 |

## Implications

- 720p is the recommended resolution for a 3 m corridor; faces clearly recognisable at 3 m, still visible to 4 m.

- UXGA adds only ~80 cm over 720p at cost of lower FPS; only use it for corridors longer than ~3.5 m.

- All thresholds drop 30–50% in dim lighting; add supplementary lighting or switch to IR camera for dark environments.

# T3b Resolution vs FPS Trade-off  [PASS]

Method: Automated res_fps_test.py. For each resolution: sends GET /res?id=<id>, waits 5 s (SETTLE_SECS), then opens one HTTP MJPEG stream. Raw bytes collected for 12 s; counts --frame boundary markers. No FPS cap. One sample JPEG per resolution saved to ./frames/. Board cold (~47–52°C).

| Resolution | Dimensions | FPS (2026-05-19) | FPS (2026-05-14) | Temp (°C) | Face range (T3) | >=20 fps? |
| --- | --- | --- | --- | --- | --- | --- |
| qvga | 320×240 | 26.5 | 25.4 | 47.3 | 100 cm | YES |
| vga | 640×480 | 25.0 | 24.8 | 50.3 | 150 cm | YES — recommended |
| svga | 800×600 | 13.3 | 13.1 | 50.3 | — | NO — hardware cliff |
| hd | 1280×720 | 13.5 | 13.9 | 52.3 | 300 cm | NO |
| sxga | 1280×1024 | — | 10.6 | — | 320 cm | NO — crashes new firmware |
| uxga | 1600×1200 | — | 7.8 | — | 380 cm | NO — crashes new firmware |

SXGA and UXGA: board crashes (resets) when switching to these resolutions in new firmware. Temp drops from 52°C to 43°C at SXGA switch = camera stopped. Old data (2026-05-14) used. Needs investigation before deploying at high resolution.

## Implications

- Hard cliff at VGA confirmed: qvga and vga deliver ~25 fps; everything above VGA drops to 13 fps. OV2640 hardware limit.

- Recommended for motion capture: VGA (640x480) — highest resolution sustaining >=20 fps.

- SVGA result (13.3 fps) confirmed consistent across both firmware versions — cliff is reproducible.

- SXGA+ crash is a regression in new firmware. Do not deploy at SXGA/UXGA until fixed.

Notes:

# T4 Power & Temperature  [PASS]

Method: Idle row automated. Stream rows captured during T5. RTSP and PIR rows require manual INA219 measurement. FPS column is lifetime average (totalFramesSent / uptime_ms) — not per-state current FPS. CPU 240 MHz

| State | Temp (°C) | FPS (lifetime avg) |
| --- | --- | --- |
| Idle | 39.3 | 0.3 |
| 1x MJPEG stream | 42.3 | 0.4 |
| 2x MJPEG streams | 43.3 | 0.7 |
| 3x MJPEG streams | 44.3 | 0.9 |
| 4x MJPEG streams | 45.3 | 1.1 |
| 5x MJPEG streams | 46.3 | 1.2 |

FPS column shows lifetime average since boot; dragged down by idle time before T5. Not comparable to old T4 FPS numbers. Use T3b (res_fps_test.py) for accurate per-resolution FPS.

## Implications

- Temperature rises ~7°C from idle to 5-stream load passive cooling sufficient in normal indoor environments.

- Board at home runs 7–10°C cooler than at work (stronger signal = less radio power = less heat).

- For accurate battery life estimate, clip INA219 on 3.3 V rail firmware estimate is not load-sensitive.

Notes:

# T5 Multiple Simultaneous Streams  [PASS]

Method: Automated opens 1–5 HTTP MJPEG streams in threads, waits 5 s per step, reads /stats. Frames rx column is cumulative total across all clients since test started NOT per-step FPS. Only the 1-client row gives a clean per-client rate.

| Clients | All alive? | Frames rx (cumulative) | Temp (°C) | Crashes |
| --- | --- | --- | --- | --- |
| 1 | Yes | 102 | 42.3 | None |
| 2 | Yes | 238 | 43.3 | None |
| 3 | Yes | 409 | 44.3 | None |
| 4 | Yes | 541 | 45.3 | None |
| 5 | Yes | 657 | 46.3 | None |

1-client clean rate: 102 frames / 5 s = ~20 fps. Beyond row 1, Frames rx includes all prior clients still running rows cannot be compared directly. All-alive column is the meaningful pass/fail metric.

## Implications

- For recognition pipeline use 1 client only gives ~20–28 fps depending on firmware and network.

- Use RTSP for recognition software and HTTP /stream for human preview only.

- Temperature at 5 clients (46°C) well below 85°C throttle threshold.

Notes:

# T6 RTSP Stream  [PASS VLC steps manual]

Method: Automated OPTIONS probe (raw TCP socket to :8554). VLC visual checks are manual.

RTSP URL: rtsp://192.168.0.39:8554/mjpeg/1  (home) / rtsp://10.10.10.155:8554/mjpeg/1  (work)

| Step | Result |
| --- | --- |
| RTSP OPTIONS probe | PASS RTSP/1.0 200 OK |
| Methods advertised | DESCRIBE, SETUP, TEARDOWN, PLAY, PAUSE |
| VLC connects and plays | PASS |
| No green bands / duplication | PASS |
| Resolution change visible in VLC | PASS |
| Quality change visible in VLC | PASS |

## Implications

- Only 1 simultaneous RTSP client supported: recognition software should be sole RTSP consumer

- Resolution changes apply immediately via /res; RTSP reflects new resolution on next VLC reconnect.

- No authentication: any device on the same network can change settings.

Notes:

# T6b 5-Minute Stability  [PASS]

Method: Automated 3 simultaneous HTTP streams held for 300 s, polled every 30 s.

| Elapsed (s) | Streams alive | Temp (°C) | Free RAM | RSSI (dBm) |
| --- | --- | --- | --- | --- |
| 30 | 3/3 | 46.3 | 189 / 323 KB | -51 |
| 60 | 3/3 | 47.3 | 188 / 323 KB | -51 |
| 90 | 3/3 | 46.3 | 181 / 323 KB | -54 |
| 120 | 3/3 | 48.3 | 188 / 323 KB | -54 |
| 150 | 3/3 | 49.3 | 188 / 323 KB | -59 |
| 180 | 3/3 | 48.3 | 188 / 323 KB | -51 |
| 210 | 3/3 | 51.3 | 181 / 323 KB | -54 |
| 241 | 3/3 | 49.3 | 190 / 323 KB | -53 |
| 271 | 3/3 | 50.3 | 188 / 323 KB | -57 |
| 301 | 3/3 | 53.3 | 188 / 323 KB | -52 |

PASS all 3 streams alive throughout. Temp plateaued at 53°C (safe limit 85°C). RAM stable, no leak. RSSI -51 to -59 dBm (good). Board runs cooler at home vs work

## Old T6b data for reference

| Elapsed (s) | Streams alive | FPS (/stats, old) | Temp (°C) | Free RAM | RSSI (dBm) |
| --- | --- | --- | --- | --- | --- |
| 30 | 3/3 | 9.2 | 56.3 | 190 / 323 KB | -38 |
| 60 | 3/3 | 6.5 | 57.3 | 183 / 323 KB | -38 |
| 120 | 3/3 | 6.5 | 59.3 | 191 / 323 KB | -39 |
| 180 | 3/3 | 6.5 | 60.3 | 182 / 323 KB | -38 |
| 300 | 3/3 | 7.0 | 62.3 | 191 / 323 KB | -38 |

Old FPS from /stats used the racy per-handler global — values were unreliable under 3-client load. Old temps higher (56–62°C) because board ran at work (weaker signal = more radio power).

## Implications

- System can run indefinitely no memory leak detected.

- Chip reaches thermal equilibrium with a heatsink

Notes:

# Test Endpoint Verification  [PASS]

Method: Automated sends GET to each control endpoint, re-reads /stats to confirm change, then restores defaults.

| Endpoint | Value set | Confirmed in /stats | Result |
| --- | --- | --- | --- |
| /res?id=qvga | qvga (320x240) | resolution = qvga | PASS |
| /quality?q=10 | q=10 (Best) | quality = 10 | PASS |
| /fps?delay=33 | 33 ms delay | frame_delay = 33 | PASS |

# Test Motion Capture Streaming Firmware  [PASS]

Method: motion_test.py 15 s MJPEG stream recorded per subject, saved as frame_XXXX.jpg to motion_captures/<subject>/. VGA 640×480, quality 20. Location: office corridor, SenSen2 WiFi.

| Subject | Face captured? | Notes |
| --- | --- | --- |
| Fast walker | Yes | Multiple clear frames mid-stride |
| Normal walker | Yes | Clear face across most frames |
| Slow walker | Yes | Many usable frames |

Key finding: 15 s at 24 fps = ~360 frames per subject. Even a fast walker crossing frame in 2–3 s produces 50+ frames guarantees at least one clear face shot.

Notes:

# Test Motion Capture: Hybrid Firmware (battery)  [PASS]

Method: motion_test.py pointing at esp32cam.local/stream. Board on power bank. Hybrid firmware: sleeps at ~14 µA until PIR fires → wakes → streams MJPEG for 30 s → sleeps.

| Subject | Face captured? |
| --- | --- |
| Fast walker | Yes |
| Normal walker | Yes |
| Slow walker | Yes |
| Wheelchair (office chair) | Yes |

## Firmware behavior

- On PIR wake: camera init → WiFi connect → mDNS (esp32cam.local) → MJPEG stream for 30 s → deep sleep.

- On 30 s timer wake: immediately deep sleep (power bank keepalive only — draws no useful current).

- Power bank issue solved: 14 µA deep sleep draw is too low for power bank auto-shutoff threshold. Timer wakeup creates brief current spike every 30 s to keep power bank alive.

Notes:

# Test Motion Capture Deep Sleep Firmware  [Partial]

Method: motion_test_deepsleep.py polls esp32cam.local/motion for up to 60 s per subject. Board wakes on PIR, takes 1 photo, serves it for 30 s, sleeps.

So this did not work as PIR scanned object off-camera or when person was turning back RTSP works better but again uses resources I have now created an alternate firmware that send multiple images instead of a simple rtsp or a single image

| Subject | Board responded? | Wake time | In-frame? | Notes |
| --- | --- | --- | --- | --- |
| Fast walker | NO |  |  |  |
| Normal walker |  |  |  |  |
| Slow walker |  |  |  |  |

## Issues found — fixes applied

Notes:

# T7 Multiple Cameras  [Not yet run; requires second unit]

Method: Manual — flash second board with same firmware, connect both to WiFi, verify independent stream and control access. Note: test scripts must have IP passed as argument (no hardcoded IP) before running T7.

| Test | Camera 1 | Camera 2 |
| --- | --- | --- |
| Both streaming simultaneously |  |  |
| Both RTSP in VLC simultaneously |  |  |
| Res change on one does not affect other |  |  |
| Motion detection independent |  |  |

──────────────────────────────────────────────────────────────────────────

Board: Seeed XIAO ESP32S3 Sense  |  Work IP: 10.10.10.155  |  Hybrid/Deepsleep: esp32cam.local